WMV. Prepay Interface and Data Structures

Rev.1.0

# Revision History

| Version | Date | Description |
| --- | --- | --- |
| 1.0 | 11.12.2024 | Initial document revision |

Table of Contents

Revision History	1

Prepay Communication Interface	2

BLE Communication Stack	2

DLMS/COSEM Application Association	3

Physical and Logical Devices	3

Minimal Set of Services	3

The Invoke_Id and Priority Byte	3

Clients	3

COSEM objects	5

SAP assignment, COSEM logical device name	5

Identification related objects	6

Security related objects	6

Clock	7

Alarms	7

Payment related objects	8

Water meter registers	9

Valve related objects	9

# Prepay Communication Interface

The meter features BLE (BlueTooth Low Energy) as local port that makes it possible to send STS tokens to the meter and read some registers relevant for prepay functionality from the meter.

# BLE Communication Stack

For the BLE interface the protocol stack looks like following:

| Application layer | DLMS/COSEM IEC 62056-5-3 |
| --- | --- |
| Convergence sublayer | IEC 62056-47 |
| BLE Peripheral | Proprietary GATT |
|  | BLE v.5.0 LL + LE PHY 2.4 GHz |

BLE data link layer uses few standard GATT profiles and one proprietary GATT profile “DLMS/COSEM”.

After connection established client must agree MTU size equal to 250 bytes.

The following services are supported:

| Service | Characteristic | Comment |
| --- | --- | --- |
| GAP (UUID=0x1800) | Device Name (UUID=0x2A00) | “WMP-0001234567” |
|  | Appearance (UUID=0x2A01) | Cat: 2-Computer, Sub: 0-Generic |
| GATT (UUID=0x1801) | Client Characteristic Configuration (UUID=0x2902) | 0x0001 - Notifications enable 0x0002 - Indications enable |
| Device Information (UUID=0x180A) | Serial Number String (UUID=0x2A25) | “1234567” |
|  | Hardware Revision String (UUID=0x2A27) | “8006” |
|  | Firmware Revision String (UUID=0x2A26) | “v.1.2.0.11” |
|  | Manufacturer Name String (UUID=0x2A29) | “ADD-TECHNOLOGY” S.R.L. |
| Current Time (UUID=0x1805) | Current Time (UUID=0x2A2B) | Read/Write current time |
| DLMS/COSEM (UUID=005a02fe-bea5-46a0-c000-73c0d9b578fc) | Metering Interface (UUID=005a02fe-bea5-46a0-c101-73c0d9b578fc) | Notify/Write DLMS/COSEM commands |

DLMS/COSEM is used as application layer protocol. This means that all application data is modeled as COSEM objects and is transmitted in DLMS formatted messages.

For detail see DLMS Blue Book Ed.15 and DLMS Green Book Ed.11.

STS tokens shall be generated in accordance with STS specifications (IEC 612055-41 Ed.3) and send to meter by invoking enter(data) method of the “Token gateway” DLMS object (see “COSEM Objects” section for detail).

# DLMS/COSEM Application Association

## Physical and Logical Devices

In DLMS/COSEM metering equipment normally operated as a COSEM server is modeled in physical and logical devices. In general, actual (physical) device can contain multiple logical devices. Water meter contains only one logical device: the management logical device (L_SAP = 1).

## Minimal Set of Services

Logical Name services are supported. Minimal set of supported application layer services:

- Block-transfer-with-get (11);

- Multiple-references (14);

- Get (19);

- Selective-Access (21);

- Action (23).

## The Invoke_Id and Priority Byte

Is handled according to DLMS Blue Book Ed.11.

When the Block Transfer is used to carry long data that do not fit into one APDU, the client must set same Invoke_Id value for all blocks.

Bit 6 (service_class) must be set by the client in order to get an answer from the meter. The meter only answers if Bit 6 is set in the request.

In the response sent from the meter Bit 6 (service _class) and Bit 7 (Priority) are irrelevant for the client, the meter must return the Invoke-Id and Priority byte as received from the client.

## Clients

The following COSEM Clients are permitted by the COSEM server:

- Public Client (L_SAP: 16),

- Prepay Client (L_SAP: 2),

| Client | Client L_SAP | Use Cases | Behavior | Services supported by a Server |
| --- | --- | --- | --- | --- |
| Public client | 16 | - Reading basic device configuration information (SAPs, COSEM logical device name, association, serial numbers, etc.). - Unicast communication with the meter. | - Accessible via BLE. - No security. - Get service only to a limited set of attributes. - Must be established by the client using the AARQ service. - ClosingBLE: on explicit closure, on timeout or on release request. | - Block-transfer-with-get. - Get. - Multiple-references. |
| Prepay Client | 2 | - Meter parameters reading. - Meter accumulated register reading. - Unicast communication with the meter. - Use Token gateway method | - Accessible via BLE. - Must be established by the client using the AARQ service with LLS or HLS (mechanism_id(5)). - Supports encryption and authentication. - Closing BLE: on explicit closure, on timeout or on release request. | - Block-transfer-with-get. - Get. - Multiple-references. - Selective access. - Action. |

Table 1

# COSEM objects

## SAP assignment, COSEM logical device name

| # | Object/Attribute Name | CL | Type | Value | Meaning | Comments | Access Rights |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | PC | PrC |  |  |  |
|  | SAP Assignment | 17 |  | 0-0:41.0.0.255 |  |  |  |  |  |  |  |
| 1 | logical_name |  | octet-string[6] | 0000290000FF |  |  | R | R |  |  |  |
| 2 | SAP_assignment_list |  | asslist_type |  |  |  | R | R |  |  |  |
|  | COSEM logical device name | 1 |  | 0-0:42.0.0.255 |  |  |  |  |  |  |  |
| 1 | logical_name |  | octet-string[6] | 00002A0000FF |  |  | R | R |  |  |  |
| 2 | value |  | octet-string[13] |  | unique identification of the logical device | Handled by the manufacture | R | R |  |  |  |
|  | Association LN for current client | 15 |  | 0-0:40.0.0.255 | Current Client Association |  |  |  |  |  |  |
| 1 | logical_name |  | octet-string[6] | 0000280001FF |  |  | R | R |  |  |  |
| 2 | object_list |  | object_list_type |  |  |  | R | R |  |  |  |
| 3 | associated_partners_id |  | associated_partners_type |  |  |  | R | R |  |  |  |
| 4 | application_context_name |  | application_context_name |  |  |  | R | R |  |  |  |
| 5 | xDLMS_context_info |  | xDLMS_context_type |  |  |  | R | R |  |  |  |
| 6 | authentication_mechanism_name |  | mechanism_name |  |  |  | R | R |  |  |  |
| 7 | LLS_secret |  | octet-string[] |  |  |  |  |  |  |  |  |
| 8 | association_status |  | enum |  |  |  | R | R |  |  |  |
| 9 | security_setup_reference |  | octet-string |  |  |  | R | R |  |  |  |
| 1 | reply_to_HLS_authentication |  |  |  |  |  |  | W |  |  |  |
|  | Association LN for Public Client | 15 |  | 0-0:40.0.1.255 | Public Client Association |  |  |  |  |  |  |
| 1 | logical_name |  | octet-string[6] | 0000280001FF |  |  | R |  |  |  |  |
| 2 | object_list |  | object_list_type |  |  |  | R |  |  |  |  |
| 3 | associated_partners_id |  | associated_partners_type |  | Management logical device (1) – public client (16) |  | R |  |  |  |  |
| 4 | application_context_name |  | application_context_name |  |  |  | R |  |  |  |  |
| 5 | xDLMS_context_info |  | xDLMS_context_type |  |  |  | R |  |  |  |  |
| 6 | authentication_mechanism_name |  | mechanism_name |  |  |  | R |  |  |  |  |
| 7 | LLS_secret |  | octet-string[] |  |  |  |  |  |  |  |  |
| 8 | association_status |  | enum |  |  |  | R |  |  |  |  |
| 9 | security_setup_reference |  | octet-string |  |  |  | R |  |  |  |  |
|  | Association LN for Prepay Client | 15 |  | 0-0:40.0.2.255 | Prepay Client Association |  |  |  |  |  |  |
| 1 | logical_name |  | octet-string[6] | 0000280002FF |  |  |  | R |  |  |  |
| 2 | object_list |  | object_list_type |  |  |  |  | R |  |  |  |
| 3 | associated_partners_id |  | associated_partners_type |  | Management logical device (1) – Prepay client (2) |  |  | R |  |  |  |
| 4 | application_context_name |  | application_context_name |  |  |  |  | R |  |  |  |
| 5 | xDLMS_context_info |  | xDLMS_context_type |  |  |  |  | R |  |  |  |
| 6 | authentication_mechanism_name |  | mechanism_name |  |  |  |  | R |  |  |  |
| 7 | LLS_secret |  | octet-string[] |  |  |  |  |  |  |  |  |
| 8 | association_status |  | enum |  |  |  |  | R |  |  |  |
| 9 | security_setup_reference |  | octet-string |  |  |  |  | R |  |  |  |
| 1 | reply_to_HLS_authentication |  |  |  |  |  |  | W |  |  |  |

## Identification related objects

| # | Object/Attribute Name | CL | Type | Value | Meaning | Comments | Access Rights |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | PC | PrC |  |  |  |
|  | Device ID 1 | 1 |  | 0-0:96.1.0.255 |  |  |  |  |  |  |  |
| 1 | logical_name |  | octet-string[6] | 0000600100FF |  |  | R | R |  |  |  |
| 2 | value |  | octet-string[] |  | Meter serial number assigned by the producer |  | R | R |  |  |  |

## Security related objects

| # | Object/Attribute Name | CL | Type | Value | Meaning | Comments | Access Rights |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | PC | PrC |  |  |  |
|  | Security Setup for Prepay Client | 64 |  | 0-0:43.0.2.255 |  |  |  |  |  |  |  |
| 1 | logical_name |  | octet-string[6] | 00002B0002FF |  |  |  | R |  |  |  |
| 2 | security_policy |  | enum |  | (0) nothing, (1) all messages to be authenticated, (2) all messages to be encrypted, (3) all messages to be authenticated and encrypted |  | R | R |  |  |  |
| 3 | security_suite |  | enum |  | (0) AES-GCM-128 for authenticated encryption and AES-128 for key wrapping |  |  | R |  |  |  |
| 4 | client_system_title |  | octet-string |  |  |  |  | R |  |  |  |
| 5 | server_system_title |  | octet-string |  |  |  |  | R |  |  |  |
|  | Frame Counter for Prepay Client | 1 |  | 0-1:43.1.2.255 |  |  |  |  |  |  |  |
| 1 | logical_name |  | octet-string[6] | 00012B0102FF |  |  | R | R |  |  |  |
| 2 | value |  | double-long-unsigned |  |  |  | R | R |  |  |  |

## Clock

| # | Object/Attribute Name | CL | Type | Value | Meaning | Comments | Access Rights |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | PC | PrC |  |  |  |
|  | Clock | 8 |  | 0-0:1.0.0.255 |  |  |  |  |  |  |  |
| 1 | logical_name |  | octet-string[6] | 0000010000FF |  |  | R | R |  |  |  |
| 2 | time |  | octet-string[12] |  | current local date and time according to technical specifications |  | R | R |  |  |  |
| 3 | time_zone |  | Long |  |  |  |  | R |  |  |  |
| 4 | status |  | Unsigned |  |  |  |  | R |  |  |  |
| 5 | daylights_savings_begin |  | octet-string[12] |  |  |  |  | R |  |  |  |
| 6 | daylights_savings_end |  | octet-string[12] |  |  |  |  | R |  |  |  |
| 7 | daylights_savings_deviation |  | integer |  |  |  |  | R |  |  |  |
| 8 | daylights_savings_enabled |  | boolean |  |  |  |  | R |  |  |  |
| 9 | clock_base |  | enum |  |  |  |  | R |  |  |  |

## Alarms

| # | Object/Attribute Name | CL | Type | Value | Meaning | Comments | Access Rights |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | PC | PrC |  |  |  |
|  | Alarms (info codes) | 3 |  | w-0:97.98.0.255 |  |  |  |  |  |  |  |
| 1 | logical_name |  | octet-string[6] | ww00616200FF | w=8 |  |  | R |  |  |  |
| 2 | Value |  | double-long-unsigned |  |  |  |  | R |  |  |  |
| 3 | scaler_unit |  | scal_unit_type | {0,255} | scaler=0, unit=255 |  |  | R |  |  |  |

## Payment related objects

| # | Object/Attribute Name | CL | Type | Value | Meaning | Comments | Access Rights |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | PC | PrC |  |  |  |
|  | Token gateway | 115 |  | 0-0:19.40.0.255 |  |  |  |  |  |  |  |
| 1 | logical_name |  | octet-string[6] | 0000132800FF |  |  |  | R |  |  |  |
| 2 | token |  | octet-string |  |  |  |  | R |  |  |  |
| 3 | token_time |  | date-time |  |  |  |  | R |  |  |  |
| 4 | token_description |  | array[] |  |  |  |  | R |  |  |  |
| 5 | token_delivery_method |  | enum |  |  |  |  | R |  |  |  |
| 6 | token_status |  | structure |  |  |  |  | R |  |  |  |
| 1 | enter (data) |  | method |  |  |  |  | A |  |  |  |
|  | Credit | 3 |  | 8-0:15.10.0.255 |  |  |  |  |  |  |  |
| 1 | logical_name |  | octet-string[6] | 08000F0A00FF |  |  |  | R |  |  |  |
| 2 | value |  | double-long |  |  |  |  | R |  |  |  |
| 3 | scaler_unit |  | scal_unit_type | {-3,19} | scaler=-3, unit=litre resolution: 000.000 l |  |  | R |  |  |  |
|  | TID | 1 |  | 0-0:96.55.5.255 |  |  |  |  |  |  |  |
| 1 | logical_name |  | octet-string[6] | 0000603705FF |  |  |  | R |  |  |  |
| 2 | value |  | double-long-unsigned |  |  | Last token identifier |  | R |  |  |  |
|  | SGC | 1 |  | 0-0:96.64.4.255 |  |  |  |  |  |  |  |
| 1 | logical_name |  | octet-string[6] | 0000604004FF |  |  |  | R |  |  |  |
| 2 | value |  | double-long-unsigned |  |  |  |  | R |  |  |  |
|  | Event Object - Payment event log | 1 |  | 0-0:96.11.13.255 |  |  |  |  |  |  |  |
| 1 | logical_name |  | octet-string[6] | 0000600B0DFF |  |  |  | R |  |  |  |
| 2 | value |  | unsigned |  | event number (0 to 255) |  |  | R |  |  |  |
|  | Payment event log | 7 |  | 0-0:99.98.13.255 |  |  |  |  |  |  |  |
| 1 | logical_name |  | octet-string[6] | 000063620DFF |  |  |  | R |  |  |  |
| 2 | buffer |  | array |  |  |  |  | R |  |  |  |
| 3 | capture_objects |  | array | {8, 0-0:1.0.0.255, 2}; {1, 0-0:96.11.13.255, 2}; {1, 0-0:96.55.5.255, 2}; {3, 8-0:15.10.0.255, 2}; | clock; event code; TID; Credit; |  |  | R |  |  |  |
| 4 | capture_period |  | double-long-unsigned | 0 | asynchronously |  |  | R |  |  |  |
| 5 | sort_method |  | enum | 1 | unsorted (FIFO) |  |  | R |  |  |  |
| 6 | sort_object |  | object definition | none | unsorted |  |  | R |  |  |  |
| 7 | entries_in_use |  | double-long-unsigned |  |  |  |  | R |  |  |  |
| 8 | profile_entries |  | double-long-unsigned | 120 |  |  |  | R |  |  |  |

## Water meter registers

| # | Object/Attribute Name | CL | Type | Value | Meaning | Comments | Access Rights |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | PC | PrC |  |  |  |
|  | Accumulated volume | 3 |  | w-0:1.0.x.255 |  |  |  |  |  |  |  |
| 1 | logical_name |  | octet-string[6] | ww000100xxFF | w=8 x=0,1 |  |  | R |  |  |  |
| 2 | value |  | long64-unsigned (for x=0) double-long-unsigned (for x=1) |  |  |  |  | R |  |  |  |
| 3 | scaler_unit |  | scal_unit_type | {-6,13} (for x=0) {-3,13} (for x=1) | scaler=-6, unit=m3 resolution: 0.000000 m3 (for x=0) scaler=-3, unit=m3 resolution: 0.000 m3 (for x=1) |  |  | R |  |  |  |

## Valve related objects

| # | Object/Attribute Name | CL | Type | Value | Meaning | Comments | Access Rights |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | PC | PrC |  |  |  |
|  | Valve status | 1 |  | w-0:96.61.7.255 |  |  |  |  |  |  |  |
| 1 | logical_name |  | octet-string[6] | ww00603D07FF | w=8 |  |  | R |  |  |  |
| 2 | value |  | unsigned |  |  | 0 – valve is closed; 100 – valve is opened; |  | R |  |  |  |